COMP_200.DOC

INFORMATION ON HOW TO CONFIGURE A CORDLESS HANDSET ON A COMPANION 200

OR A MERIDIAN 1 (IN SKINS!)

FROM THE CUSTOMER ADMIN (NORSTAR) TERMINAL WHICH SHOULD BE

LOCATED BY THE CPU. LOG INTO THE TERMINAL TO CHECK FOR SPARE

EXTNS & ENABLE THE REGISTRATION PORT.

(CUSTOMER ACCESS LEVEL)    NOTE! USE THIS LEVEL FIRST.

PRESS THE "FEATURE" BUTTON

FOLLOWED BY **9                                DISPLAY SHOWS "Passcode"

KEY IN PASSCODE    (DEFAULT = 9999 OR COULD BE 23646)

(Spells ADMIN)

DISPLAY SHOWS "1. Registration"

KEY "SHOW" OR "ARROW DOWN" BUTTON

DISPLAY SHOWS "Mstr Reg Enbl: N"

CHANGE

KEY "CHANGE" BUTTON IF REQUIRED TO "Y"    FOR YES

(THIS IS TO ALLOW HANDSET PROGRAMMING ACCESS TO CONTROL UNIT!)

KEY "NEXT" OR "-->" BUTTON

DISPLAY SHOWS "WTN Reg. Status"

KEY "SHOW" OR "ARROW DOWN" BUTTON

DISPLAY SHOWS "Show WTN: _"

KEY "SHOW" OR "ARROW DOWN" BUTTON

(IF PORT IS USED!)                DISPLAY SHOWS "W0500:registered"

DEREG

(OF IF PORT IS SPARE!)        DISPLAY SHOWS "W0500:available"

KEY "NEXT" OR "-->" BUTTON IF REQD UNTIL A SPARE PORT IS FREE.

I.E "0513" CONFIGURE OR COPY A 500 SET AS E.G... TN 05 13    ON THE

MERIDIAN MAKING SURE ON V21 ONWARDS THAT "WRLS" = YES & CLS CNDA

IS CONFIGURED.

IF THE 500 SET IS TO BE LINKED FROM ANOTHER SET I.E... M2616D

THEN MAKE SURE THAT ALL FORWARDING IS REMOVED FROM THE 500 SET AS

IT CAN CAUSE PROBLEMS.

WHEN YOU HAVE FINISHED PROGRAMMING ON THE "NORSTAR" ADMIN

TERMINAL, PRESS THE RED "RLS" KEY.

DISPLAY SHOWS "End of session"

(INSTALLER ACCESS LEVEL) NOTE!    USE IF CUSTOMER CODE IS WRONG

PRESS THE "FEATURE" BUTTON

FOLLOWED BY **0                                DISPLAY SHOWS "Passcode"

KEY IN PASSCODE    (DEFAULT = 0000 OR COULD BE 266344)

(Spells CONFIG)

DISPLAY SHOWS "A.Configuration"

KEY "NEXT" OR "-->" BUTTON

DISPLAY SHOWS "B.Administration"

KEY "SHOW" OR "ARROW DOWN" BUTTON

DISPLAY SHOWS "1. Registration"

KEY "SHOW" OR "ARROW DOWN" BUTTON

& CARRY ON PROGRAMMING AS SHOWN IN CUSTOMER ACCESS LEVEL.