# DECT SERIAL CONNECTION PROCEDURE

Serial Connectivity:

- Remove the Ethernet D/B and ensure the Jumper pins on board are to V.24

- This procedure is based on Windows 2000

- Settings – Control Panel – Phones&Modems

- Click Add and Select the following:

- Do not detect Modem

- Standard Modem – Connection between 2 computers

- Free Comm Port, 2 used in this example

- Go to properties of this connection and ensure the following is used:

  - 38,400 and in advanced: flow control none, 8 none 1

- Go to Network Dial up and Connection and do the following:

  - Make a new connection

  - Connect direct to another computer

  - Guest

  - Select Comm Port (i.e. 2)

  - For All Users

  - Give it a name (DECT SERIAL)

- Properties of the dial up connection just made in step 6

  - Comms between 2 computers configure 38,400

  - In networking tab change TCP/IP to the following IP Address 192.168.1.200 (REMEMBER THIS IP ADDRESS FOR LATER)

- Double Click direct connection, click connect (no name/password)

- WAIT

- When connected double click the computer icon (bottom right of pc) in the details window of this it will show the IP ADDRESS OF THE DECT CARD!!!!!

Serial connection once you have the IP Address:

- Go to OTM

- System Properties:

  - Communications, add, ppp (point this at comm2), applications DECT check that the communications is PPP on comm Port 2

- Open DECT

- File Add, use same procedure as normal except in the Comms Tab insert the IP address created in previous procedure (192.168.1.200) and ensure you click Serial

- Serial Details: Ensure you input IP address 192.168.1.200 and apply this to comm Port 2

- Connect.